HOME SERVER

Complete Reference Guide

Dell OptiPlex 7050 | Ubuntu Server 24.04 LTS

Last updated: March 27, 2026

# 1. Hardware Specifications

## 1.1 Server Machine

| Component | Details |
| --- | --- |
| Model | Dell OptiPlex 7050 Mini Tower |
| Processor | Intel Core i7-6700 @ 3.40 GHz (4 cores, 8 threads) |
| RAM | 16 GB DDR4 (max supported: 64 GB) |
| Storage (3.5" bay) | Seagate BarraCuda 1TB, 7200 RPM, SATA (ST1000DM010) |
| Storage (2.5" bay) | Empty - planned for SSD boot drive |
| Optical Drive | Removed (bay available for adapter) |
| BIOS Version | Dell Inc. 1.5.2 (June 19, 2017) |
| Form Factor | Mini Tower |
| Power Consumption | ~40W idle |

## 1.2 BIOS Configuration

| Setting | Value |
| --- | --- |
| SATA Operation | AHCI (changed from RAID) |
| Secure Boot | Disabled |
| Boot Mode | UEFI |
| Boot Sequence | Ubuntu (primary), Windows Boot Manager (disabled) |

## 1.3 Donor Machine

Identical Dell OptiPlex 7050, stripped of RAM (moved to server). Kept assembled as a parts shelf for replacements: PSU, CPU cooler, case fans, SATA cables, blue drive caddy.

# 2. Network Configuration

## 2.1 Local Network

| Setting | Value |
| --- | --- |
| ISP | HOT (Gigabit Fiber) |
| Router | HOT Box 5F |
| Router Admin | 192.168.1.1 |
| Server Static IP | 192.168.1.88 |
| Server MAC Address | 8C:EC:4B:43:6B:7D |
| Hostname | mna-server |
| DHCP Lease | Static - MAC bound to 192.168.1.88 |

## 2.2 Tailscale VPN

| Setting | Value |
| --- | --- |
| Tailscale IP | 100.86.110.81 |
| Plan | Free tier |
| Connected Devices | Server, Android phone, Windows PC |
| Purpose | Encrypted remote access from anywhere |

## 2.3 Firewall (UFW)

Firewall is active with the following rules:

| Rule | Description |
| --- | --- |
| Default deny incoming | All incoming traffic blocked unless explicitly allowed |
| Default allow outgoing | All outgoing traffic permitted |
| Allow on tailscale0 | All traffic over Tailscale VPN allowed |
| Allow on eth0 from 192.168.1.0/24 | Local network traffic allowed |
| Allow Samba | SMB file sharing ports open |

## 2.4 Remote Access Chain

From any device with Tailscale connected, the access path is:

Termux (Android) / Windows Terminal (PC)

|

v

Mosh or SSH (over Tailscale VPN to 100.86.110.81)

|

v

tmux (persistent session 'main' on server)

|

v

Claude Code / Shell / Services

All traffic is encrypted end-to-end via Tailscale. No ports are exposed to the public internet. Mosh handles connection drops gracefully. tmux keeps sessions alive indefinitely.

### Quick Connect Shortcuts

| Device | Command | Setup Location |
| --- | --- | --- |
| Android (Termux) | dev | ~/.bashrc alias |
| Windows PC | dev | PowerShell $PROFILE function |

### Android Shortcut

# In ~/.bashrc on Termux:

alias dev='mosh mna@100.86.110.81 -- tmux attach -t main'

### Windows Shortcut

# In PowerShell $PROFILE:

function dev { ssh mna@100.86.110.81 -t "tmux attach -t main" }

### SSH Key Authentication

SSH keys are configured for passwordless access. Keys were generated on the Android phone using ed25519 and copied to the server via ssh-copy-id.

# 3. Operating System

## 3.1 Base System

| Setting | Value |
| --- | --- |
| OS | Ubuntu Server 24.04 LTS |
| Install Type | Full (not minimized) |
| Filesystem | ext4 (no LVM) |
| Boot Mode | UEFI with GPT partitioning |
| SSH Server | OpenSSH (installed during setup) |
| Username | mna |
| Server Hostname | mna-server |

## 3.2 Installed Packages

| Package | Purpose |
| --- | --- |
| tmux | Persistent terminal sessions |
| mosh | Mobile-friendly SSH replacement |
| curl | HTTP requests and downloads |
| git | Version control |
| htop | System resource monitor |
| wget | File downloads |
| net-tools | Network utilities (ifconfig, etc.) |
| samba | Windows-compatible network file sharing |
| docker | Container runtime (installed via get.docker.com) |
| docker compose | Multi-container orchestration (bundled with Docker) |
| nodejs 22.x | JavaScript runtime for Claude Code |
| claude-code | Anthropic AI coding assistant (npm global) |
| tailscale | VPN mesh network client |
| ufw | Firewall management |

## 3.3 Key Configuration Files

| File | Purpose |
| --- | --- |
| /etc/ssh/sshd_config | SSH server configuration (PasswordAuthentication yes) |
| /etc/samba/smb.conf | Samba file share definitions |
| ~/.bashrc | Shell config (API key, aliases, default directory) |
| ~/docker/docker-compose.yml | All Docker service definitions |

# 4. Docker Services

All services are defined in ~/docker/docker-compose.yml and run on a shared Docker network called 'monitoring'. All services have restart: unless-stopped policy.

## 4.1 Service Overview

| Service | Image | Port | Purpose |
| --- | --- | --- | --- |
| Portainer | portainer/portainer-ce | 9443 (HTTPS) | Docker management web UI |
| Jellyfin | jellyfin/jellyfin | 8096 | Media server (video, photos, music) |
| n8n | n8nio/n8n | 5678 | Workflow automation |
| Syncthing | syncthing/syncthing | 8384 | File sync between devices |
| Filebrowser | filebrowser/filebrowser | 8080 | Web-based file manager |
| Uptime Kuma | louislam/uptime-kuma | 3001 | Service monitoring dashboard |
| Watchtower | containrrr/watchtower | none | Auto-update containers daily at 4 AM |

## 4.2 Access URLs

### From home network (192.168.1.x)

| Service | URL |
| --- | --- |
| Portainer | https://192.168.1.88:9443 |
| Jellyfin | http://192.168.1.88:8096 |
| n8n | http://192.168.1.88:5678 |
| Syncthing | http://192.168.1.88:8384 |
| Filebrowser | http://192.168.1.88:8080 |
| Uptime Kuma | http://192.168.1.88:3001 |

### From anywhere via Tailscale

| Service | URL |
| --- | --- |
| Portainer | https://100.86.110.81:9443 |
| Jellyfin | http://100.86.110.81:8096 |
| n8n | http://100.86.110.81:5678 |
| Syncthing | http://100.86.110.81:8384 |
| Filebrowser | http://100.86.110.81:8080 |
| Uptime Kuma | http://100.86.110.81:3001 |

## 4.3 Service Details

### Portainer

Docker container management web UI. Allows starting, stopping, and monitoring all containers through a browser. Runs on HTTPS with a self-signed certificate.

Volume: ~/docker/portainer:/data

Volume: /var/run/docker.sock:/var/run/docker.sock

### Jellyfin

Media server for streaming video, photos, and music. Install the Jellyfin app on phone and PC for the best experience. Libraries are configured through the web UI dashboard. Media files go in ~/media/ subfolders (movies, tv, music, photos).

Volume: ~/docker/jellyfin:/config

Volume: ~/media:/media

### n8n

Workflow automation platform. Used for business automations including order confirmation emails for Table Tops By Nomi. The N8N_SECURE_COOKIE=false environment variable is set to allow HTTP access.

Volume: ~/docker/n8n:/home/node/.n8n

Note: Data directory must be owned by UID 1000 (sudo chown -R 1000:1000 ~/docker/n8n)

### Syncthing

Continuous file synchronization between server, phone, and PC. Phone photos are configured to sync automatically to the server. The server Syncthing identification code is YIDDMIJ (short form).

Volume: ~/docker/syncthing:/var/syncthing

Ports: 8384 (web UI), 22000 (sync TCP/UDP), 21027 (discovery UDP)

### Filebrowser

Web-based file manager for browsing, uploading, and downloading files on the server through a browser. Mounted to the home directory so all files are accessible.

Volume: ~/:/srv

Volume: ~/docker/filebrowser:/config

Default login: admin (password was auto-generated, check docker logs filebrowser)

### Uptime Kuma

Monitoring dashboard that checks all services every 60 seconds and displays uptime status. Monitors are configured using container names as hostnames on the shared Docker network.

Volume: ~/docker/uptimekuma:/app/data

Monitors: jellyfin:8096, n8n:5678, filebrowser:80, syncthing:8384, portainer:9443

### Watchtower

Automatically checks for updated Docker images and pulls them. Runs daily at 4 AM. Cleans up old images after updating.

Volume: /var/run/docker.sock:/var/run/docker.sock

Schedule: 0 0 4 * * * (4:00 AM daily)

# 5. File Sharing and Storage

## 5.1 Directory Structure

| Directory | Purpose |
| --- | --- |
| ~/media/movies | Movie files for Jellyfin |
| ~/media/tv | TV show files for Jellyfin |
| ~/media/music | Music files for Jellyfin |
| ~/media/photos | Photos for Jellyfin |
| ~/documents | Personal documents |
| ~/backups | Backup storage |
| ~/docker | Docker configs and service data |
| ~/projects | Claude Code working directory |

## 5.2 Samba (Windows File Sharing)

Samba provides Windows-compatible network shares accessible from File Explorer on the PC.

### Configured Shares

| Share Name | Path | Access |
| --- | --- | --- |
| media | /home/mna/media | Read/Write - user mna |
| documents | /home/mna/documents | Read/Write - user mna |
| home | /home/mna | Read/Write - user mna (full home directory) |

### Accessing from Windows

Open File Explorer and type in the address bar:

\\192.168.1.88

Or map a specific share as a network drive:

\\192.168.1.88\media

Credentials: username mna, password is the Samba password (set separately from Linux login via sudo smbpasswd -a mna).

## 5.3 Syncthing File Sync

Syncthing syncs files between the server and phone automatically over the network. Phone photos are configured to sync from the camera folder to the server.

| Sync Pair | Details |
| --- | --- |
| Phone Photos | DCIM/Camera on phone syncs to /var/syncthing/photos on server |
| Server Role | Receive Only (for photos) - prevents accidental deletion |

# 6. Claude Code

## 6.1 Installation

| Component | Details |
| --- | --- |
| Node.js | v22.x (installed via nodesource) |
| Claude Code | Installed globally via npm install -g @anthropic-ai/claude-code |
| API Key | Set as ANTHROPIC_API_KEY in ~/.bashrc |
| Working Directory | ~/projects |
| CLAUDE.md | Initialized via /init command in ~/projects |

## 6.2 Usage

Claude Code runs natively on the server (not in Docker). Access it through the remote access chain:

# From phone (Termux):

dev

# From PC (Windows Terminal):

dev

# Then inside tmux:

cd ~/projects

claude

## 6.3 tmux Session Management

| Action | Keys/Command |
| --- | --- |
| New session | tmux new -s main |
| Attach to session | tmux attach -t main |
| Detach (keep alive) | Ctrl+B, then D |
| New window | Ctrl+B, then C |
| Switch window | Ctrl+B, then window number (0, 1, 2...) |
| List windows | Ctrl+B, then W |
| Scroll mode | Ctrl+B, then [ (exit with Q) |
| Close terminal | Just close it - tmux session persists |

# 7. Maintenance and Commands

## 7.1 Common Docker Commands

| Command | What it does |
| --- | --- |
| cd ~/docker && docker compose ps | Show status of all containers |
| docker compose up -d | Start all services |
| docker compose down | Stop all services |
| docker compose restart <service> | Restart a specific service |
| docker compose pull | Pull latest images |
| docker logs <container> | View container logs |
| docker compose up -d <service> | Start a specific service |

## 7.2 System Maintenance

| Command | What it does |
| --- | --- |
| sudo apt update && sudo apt upgrade -y | Update system packages |
| sudo reboot | Reboot the server |
| htop | Interactive system resource monitor |
| df -h | Check disk space usage |
| sudo ufw status | Check firewall rules |
| sudo systemctl status ssh | Check SSH service status |
| sudo systemctl restart smbd | Restart Samba file sharing |
| tailscale status | Check Tailscale connection |
| tailscale ip | Show Tailscale IP address |

## 7.3 Troubleshooting

### Service not loading in browser

Check if the container is running: docker compose ps (from ~/docker). If it shows as exited or restarting, check the logs: docker logs <container name>. Common fix: restart with docker compose restart <service>.

### Cannot connect remotely

Verify Tailscale is running on both devices. On the server: tailscale status. On your phone/PC: check the Tailscale app is connected. If Tailscale is up but SSH fails, check the firewall: sudo ufw status.

### Permission denied errors in Docker

Some containers run as specific users. For n8n, the data directory must be owned by UID 1000: sudo chown -R 1000:1000 ~/docker/n8n. For other containers, check the documentation for the required user.

### Forgot a service password

Stop the container, delete its database file (with sudo rm if needed), and restart. The service will prompt for a new account setup. Check docker logs for auto-generated passwords (Filebrowser does this).

# 8. Future Upgrades

## 8.1 Planned

| Item | Purpose | Est. Cost (ILS) |
| --- | --- | --- |
| 2.5" SATA SSD (256GB) | Boot drive for faster OS performance | 60-80 |
| UPS (600-800VA) | Power protection and clean shutdown | 200-300 |
| 4-8TB HDD | Media library expansion | 300-600 |
| USB 3.0 HDD Enclosure | Use second BarraCuda as extra storage | 30-50 |

## 8.2 Optional Software

| Service | Purpose |
| --- | --- |
| Nginx Proxy Manager | Clean URLs for services (e.g., jellyfin.local) |
| Restic / Duplicati | Automated backups to local drive or cloud (Backblaze B2) |
| Backblaze B2 | Offsite cloud backup (~$5/TB/month) |
| apcupsd / NUT | UPS monitoring and auto-shutdown on power loss |

## 8.3 Estimated Monthly Costs

| Item | Cost |
| --- | --- |
| Server electricity (~40W idle) | 15-17 ILS (~$4-5) |
| Internet (already paying) | No additional cost |
| Tailscale (free tier) | Free |
| Cloud backup (optional) | ~$5/TB/month |
| Total | 15-17 ILS/month + optional backup |